Learning Guide from Work Method from Lesson 3

Index

What you will find in this guide…	3

Which videos do I need to review?	4

What is the room like?	4

What do these videos look like?	5

Why have videos where you only see the 1st minute, and videos where you see the whole simulation?	6

What images from different perspectives in the room are these?	6

Images of flow lines for simulations with and without ventilation	8

How should I analyze the videos?	8

PAFSE Project: Scenario 1: Droplets & the Physics of Viruses Transmission

LEARNING GUIDE FROM CLASS 3

Analysis of Simulations from class 3

Sustainable Development Goals

Domains of Citizenship Education

Sustainable Development

Health

Competencies when finishing Compulsory Schooling

Critical and creative thinking, well-being, health and environment

Scientific, technical and technological knowledge

Languages and texts

Information and Communication

# What you will find in this guide…

In this script you will find the study method proposed for class 3, that is, how you should analyze, first, the series of videos we have prepared for you, and then, the steps you should follow to answer the worksheet of this class!

# Which videos do I need to review?

During this class you will have to analyze a series of videos that represent CFD (Computational Fluid Dynamics) simulations of the distribution of respiratory particles in a room, emitted in different breathing regimes. The three simulated breathing regimes were as follows:

- Situation where a person in the room is talking;

- Situation where a person in the room is sneezing;

- Situation where a person in the room is coughing.

In situations 1,2 and 3, the room will have a ventilation system. You will also have to analyze a fourth situation: when a person is talking in a room without a ventilation system.

## What is the room like?

In the videos of class 3, for all simulated breathing regimes, the elements that make up the room are the same, being the ones you see in Figure 1 illustrated below.

Figure 1. Room Caption

As shown in Figure 1, the room where the simulations were carried out consists of four occupants, six lamps (rectangles on the ceiling), and a ventilation system composed of two diffusers installed on the floor and one extractor placed on the ceiling. The diffusers will inject the air into the room, and the extractor will remove the air from the room.

As already mentioned, the room will always have these elements whether the simulation is in the situation in which the occupant speaks, sneezes, or coughs, except for the situation in which the occupant speaks in a room without a ventilation system. In this case, the room will have neither diffusers nor the extractor.

## What do these videos look like?

As already mentioned, you will have to analyze videos where you will observe the distribution of respiratory particles emitted in four different situations:

- Situation in which a person speaks;

- Situation in which a person sneezes;

- Situation in which a person coughs;

- Situation in which a person speaks without a ventilation system.

The room where situations 1, 2, 3 and 4 were simulated will always have the elements of Figure 1.

To analyze each of these simulations you will have at your disposal three elements:

- Videos in which you see what happens in each room, but only in the 1st minute of the simulation.

- Videos in which you see what happens in each room throughout the whole simulation.

- Images of the rooms from different perspectives, at the end of the simulation.

Important Note:

In all simulations, only occupant 1 observed in Figure 1 emitted particles. No other occupants emitted particles!

Each simulation was conducted for a total time of 60 minutes. In the videos you can always see the simulation time in the upper left corner of each video, as you can see in Figure 2.

In each video you will also see, in the lower right corner, a color caption that represents, in meters, the size of each particle emitted by the occupant 1. The scale goes from 1∙10-6 m or 0.0000001 m (blue particles), to 1∙10-3m or 0.0001 m (orange/red particles), as shown in Figure 2.

Figure 2. Simulation Time and Video Subtitles

### Why have videos where you only see the 1st minute, and videos where you see the whole simulation?

Two types of videos were created to help you analyze them. That is, in the videos in which only the 1st minute of the simulation is seen, you will be able to see, in detail, the beginning of the path traveled by the particles after they are emitted.

With the total simulation video, you will be able to observe, in general, what was happening to the particles throughout the simulation.

### What images from different perspectives in the room are these?

Sometimes the results at the end of the simulations may not be very easy to analyze because the perspective of the room does not allow you to observe everything that happened in the simulations. To solve this problem, a set of images was created at the end of each simulation (at 60 minutes), from different perspectives of the room. That is to say, if you have a problem analyzing any detail in the videos, you can always use these images to help you understand what you wanted. In the image of Figure 3 you have, as an example, several perspectives of the room at the end of the simulation in which occupant 1 spoke.

Figure 3. Images from different perspectives of the room: speaking simulation

### Images of flow lines for simulations with and without ventilation

In addition to these images, you will also have at your disposal two images related to the air flow lines developed in the room, when it has a ventilation system and when it does not have a ventilation system.

The images are the ones you see in Figure 4 depicted below. The flow lines, as you saw in class 1, are the lines that allow you to understand how the air in the room is distributed. These images can help you better interpret the phenomena that occurred in each simulation.

Figure 4. Room air flow lines with ventilation (left) and without ventilation (right)

# How should I analyze the videos?

As already seen, you will have at your disposal two types of videos: one in which you see only the 1st minute of the simulation, and a second type in which you see the entire simulation. The recommended sequence you should follow to review the videos is as follows:

- First, you should analyze the simulation videos with only the 1st minute for the four situations. Preferably, start by analyzing the situation of talking with and without ventilation, then, that is, if you analyze the video in which the occupant speaks with ventilation, analyze the video in which occupant 1 speaks in the room without a ventilation system;

- After analyzing the videos of the 1st minute of the simulation for the three situations, analyze the videos of the total simulation for the three situations;

- Finally, and only if you need to, analyze the images from the different perspectives of the room, at the end of each simulation.